Safe Work Method Statement

Job:.	Handling hire trailers and house floats		  Client:  Billy Thorpe

Location:	415 Johns Street Bayswater……..

Date: . 21/07/2007				Project Duration:  27 / 07 /  2007	to	29 / 07 /  2007

Principle Contractor:

Prepared By:   John

Consultation with: 	 Phillip, & 	Trish

| Tools and Equipment required | Personal Protective Equipment | Skills and Competencies Required |
| --- | --- | --- |
| - Jockey wheel | - Gloves - High Visibility Vest (Day/Night) | - Manual Handling |

SAFE WORK METHOD STATEMENT

Job:  Handling hire trailers and house floats           Client:  ......................................................................................

| STEP BY STEP WORK ACTIVITY | POTENTIAL HAZARD/RISK What are the potential hazards that could cause a safety incident or injury? | CONTROL MEASURES - Elimination – eliminate the work practice, plant and/or equipment responsible for the hazard. - Substitution – substitute the work practice, plant and/or equipment for a safer alternative. - Engineering control – redesign the work practice, use of materials, plant or equipment to attain a safer alternative. - Administrative Control – change the procedures to reduce exposure (job rotation, training etc) - Protective Equipment for Personnel – select appropriate equipment/apparel to reduce risk/exposure. | Name of responsible person to action |
| --- | --- | --- | --- |
| 1. Clear work area. | - Struck by vehicle - Slips trips and falls - Extreme weather conditions | - Observe vehicle movement on the forecourt around the trailer/horse float parking area. - Ensure there is no evidence of spills in the area - Step carefully over any curbs or uneven surfaces - Relocate objects or vehicles that could impede movement of the trailer or horse float. |  |
| 2. Prepare the trailer or horse float. | - Manual force required to lift the drawbar. - Force to reposition the trailer or horse float to align it with the vehicle - Dropping or rolling the drawbar/jockey wheel on the foot. - Jamming fingers in the coupling or safety chains on the drawbar. | - Wear enclosed footwear at all times - Use a jockey wheel to raise / lower drawbar - Use chocks behind the wheels to eliminate the trailer or horse float rolling backwards - Reverse the vehicle to the drawbar rather than manually pull the trailer or float to the car - Wear gloves to protect the fingers from grease and damage - Regularly lubricate the couplings to ensure minimal force to operate |  |
| 3. Attaching to vehicle | - Entrapment of fingers - Manual force to position coupling onto tow bar - Rolling the jockey wheel onto the feet | - Use the jockey wheel to elevate the drawbar and to lower it onto the tow bar - Seek assistance if fine movement of the trailer or horse float is required to accurately position it for coupling - Use a “straddle stance” lifting technique at the lifting point to balance the body and keep the feet away from the jockey wheel |  |
| 4. Reversing the trailer or horse float | - Struck by vehicle - Damage to property | - Before reversing ensure a “spotter” is located with clear view behind the horse float or trailer. - Clear the parking area of obstructions - Check overhead clearance for tall trailer canopies and horse floats |  |