Safe Work Method Statement

Job:.	Tank Dipping		  Client:  Billy Thorpe

Location:	415 Johns Street Bayswater……..

Date: .21 / 07 /  2007					 	Project Duration:  27 / 07 /  2007	to	29 / 07 /  2007

Principle Contractor:

Prepared By:  _John

Consultation with: 	Phillip, & 	Trish

| Tools and Equipment required | Personal Protective Equipment | Skills and Competencies Required |
| --- | --- | --- |
| - Spill kit wheelie bin with reflective tape - flameproof torch - e-flare | - Gloves - High Visibility Vest (Day/Night) | - First aid - Manual Handling |

SAFE WORK METHOD STATEMENT

Job:.............................................................................................................................  Client:  ......................................................................................

| STEP BY STEP WORK ACTIVITY | POTENTIAL HAZARD/RISK What are the potential hazards that could cause a safety incident or injury? | CONTROL MEASURES - Elimination – eliminate the work practice, plant and/or equipment responsible for the hazard. - Substitution – substitute the work practice, plant and/or equipment for a safer alternative. - Engineering control – redesign the work practice, use of materials, plant or equipment to attain a safer alternative. - Administrative Control – change the procedures to reduce exposure (job rotation, training etc) - Protective Equipment for Personnel – select appropriate equipment/apparel to reduce risk/exposure. | Name of responsible person to action |
| --- | --- | --- | --- |
| 1. Cordon off work area | - Struck by vehicle - Slips trips and falls - Extreme weather conditions | - Observe vehicle movement on the forecourt - Ensure there is no evidence of a spill within the work area - Step carefully over any curbs or uneven surfaces - Place the wheelie bin within 2m of the dipping point - Place the e-flare on top of the wheelie bin and turn the e-flare on |  |
| 2. Remove the dip pint cover plate and cap | - Back strain - Inhalation of fumes - Tank pressurization if fuel delivery is occurring simultaneously - Water accumulated in dip point enters dip channel | - Face oncoming traffic- observe traffic movement on the forecourt - Work within the confines of the cordoned area - Squat to remove the dip point cover and keep back as straight as possible - Be upwind to minimize risk of vapour inhalation - Do not dip when fuel delivery is in progress - Scoop any water accumulated within the dip point prior to removing dip point cap |  |
| 3. Remove dip stick and wipe clean with paper towel | - Overhead hazard - Fuel contact with skin | - Confirm no overhead hazards within striking distance of the dip point - PPE- gloves |  |
| 4. Place water finding paste onto the lower end of the dip stick Approximately 50mm from the bottom | - Water finding paste contact with skin | - PPE- gloves |  |
| 5. Lower the dipstick into dip point | - Damage to bottom of underground tank | - Gently lower dip stick into dip point - Use hand over hand technique |  |
| 6. Remove dipstick. Read and record quantity of fuel and water | - Poor visibility - Fuel contact with skin | - Use flameproof torch - PPE- gloves |  |
| 7. Wipe the dipstick and dip a second time Use the average between these two dip readings as the recorded product level | - Overhead hazard - Fuel contact with skin | - Confirm no overhead hazards within striking distance of the dip point - PPE- gloves |  |
| 8. Replace dip point cap and cover plate | - Back strain - Pinch points | - Face oncoming traffic - Work within the confines of the cordoned area - Squat down to replace dip point cover plate and keep back as straight as possible - Stay away from pinch points on dip cap, push down with flat hand |  |
| 9. Return equipment to showroom | - Struck by vehicle - Manual handling - Waste materials | - Observe traffic movement - Use correct lifting technique when collecting cones - Step over kerbing carefully - Dispose of paper towels in external waste bin |  |